# Buick V6 Casting Numbers and Internal Specs For Most Years
Compiled by Joel Vande Berg

| Year | CID | Liter | Bore | Stroke | Description | Block Casting | Crank Forging | Head Casting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1982 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Turbo Charged, Crank may have 5/8" or 3/4" Pulley Bolt Hole | 25506818 | 1260877 |  |
|  |  |  |  |  |  | 25514290 |  |  |
| 1981-79 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Turbo Charged, Crank may have 5/8" or 3/4" Pulley Bolt Hole | 25506818 | 1260877 | 25506293 |
|  |  |  |  |  |  | 25506397 |  | 25504443 |
| 1978 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Turbo Charged, Crank may have 5/8" or 3/4" Pulley Bolt Hole | 1254083 | 1260877 | 1257051 |
|  |  |  |  |  |  | 1261438 | 1257125 | 1257722 |
|  |  |  |  |  |  |  | 1255674 |  |
| 1982 | 231 | 3.8 | 3.80 | 3.40 | V6, Even Fire, Crank may have 5/8" or 3/4" Pulley Bolt Hole | 25506818 | 1260877 |  |
|  |  |  |  |  |  | 25514290 |  |  |
| 1981-79 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Crank may have 5/8" or 3/4" Pulley Bolt Hole | 25506818 | 1260877 | 25506293 |
|  |  |  |  |  |  | 25506397 | 1257125 | 25504443 |
|  |  |  |  |  |  | 25505554 | 1255674 | 1262098 |
|  |  |  |  |  |  | 25514290 |  | 1261377 |
|  |  |  |  |  |  | 1261787 |  |  |
| 1978-77 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Split Rod Pin Crank, Even Fire Engine | 1254083 | 1257125 | 1257051 |
|  |  |  |  |  |  | 1255862 | 1260877 | 1257661 |
|  |  |  |  |  |  | 1261438 | 1255674 | 1257722 |
| 1977 | 231 | 3.8 | 3.80 | 3.40 | V6, Even-Fire, Straight Rod Pin Crank, Odd Fire Engine w/Integrated Dist. Drive | 1254083 | 1255645 | 1257051 |
|  |  |  |  |  |  | 1255862 | 1255846 | 1257661 |
|  |  |  |  |  |  | 1261438 | 1357898 | 1257722 |
|  |  |  |  |  |  |  | 1378354 |  |
| 1976-75 | 231 | 3.8 | 3.80 | 3.40 | V6, Odd-Fire, Straight Rod Pin Crank, Odd Fire Engine w/Non-Integrated Dist. Drive | 1249432 | 1357898 | 1249175 |
|  |  |  |  |  |  |  | 1378351 | 1254164 |
|  |  |  |  |  |  |  | 1378354 |  |
| 1967-74 | 225 | 3.7 | 3.75 | 3.40 | V6, Odd-Fire, Kaiser Jeep Corp/AMC | 1358435 | 1357898 | 1358676 |
|  |  |  |  |  |  | 1396736 |  | 1368767 |
|  |  |  |  |  |  | 1381623 |  |  |
|  |  |  |  |  |  | 1375162 |  |  |
|  |  |  |  |  |  | 991503 |  |  |
| 1963-65 | 225 | 3.7 | 3.75 | 3.40 | V6, Odd-Fire, FWD/RWD w/one barrel Carb | 1396736 | 1327898 | 1358767 |
| 1961-63 | 198 |  |  |  | V6, Odd-Fire, FWD/RWD Buick applications only | 1349430 |  |  |

Block, Head, Crank - Part Numbers

| Application | Year | Block | Head | Crank |
| --- | --- | --- | --- | --- |
| 3.8L RWD | 1975-76 | 1249432 | . | . |
| 3.8L RWD | 1977 | 1254083 | 1237722 | 1257125 |
| . | . | 1255862 | . | . |
| . | . | 1232438 | . | . |
| 3.8L RWD | 1978 | 1261438 | 1257661 | 1260877 |
| . | . | 1261787 | 1257051 | 2551229 |
| . | . | 25505554 | . | . |
| Turbo RWD | . | 1261438 | 1257661 | Rolled Fillets |
| 3.8L RWD | 1979 | 1261787 | 1262098 | 25500229 |
| . | . | 25505554 | 25504443 | . |
| Turbo RWD | . | 25514290 | 25504443 | . |
| . | . | 25505554 | 25504443 | Rolled Fillets |
| 3.8L RWD | 1980-81 | 25506818 | 25505900 | See 1978 |
|  | . | . | 25506293 | . |
| Turbo Monte/Regal | . | 25506818 | 25505900 | See 1978 |
|  | . | . | 25506293 | . |
| Turbo Riviera | . | 25506818 | 25505900 | See 1978 |
|  | . | 25506485 | 25506293 | . |
| 3.8L RWD | 1982 | 25506818 | 25510191 | See 1978 |
| . | . | . | 25514290 | . |
| Turbo Regal | . | 25506818 | 25510191 | See 1978 |
| . | . | . | 25514290 | . |
| Turbo Riviera | . | 25506481 | 25510191 | See 1978 |
| . | . | 25524289 | . | . |
| 3.8L RWD | 1983 | 25524289 | 25518445 | See 1978 |
|  |  | 25506397 | . | . |
| Turbo Regal | . | 25524289 | 25518445 | See 1978 |
| Turbo Riviera | . | 25524289 | 25518445 | See 1978 |
| 3.8L RWD | 1984 | 25524290 | 25518445 | See 1978 |
| . | . | 25528601 | . | . |
| . | . | 25520040 | . | . |
| Turbo Regal | . | 25520040 | 25518445 | See 1978 |
| . | . | 25518603 | . | . |
| Turbo Riviera | . | 25520040 | 25518445 | See 1978 |
| . | . | 25520042 | . | . |
| . | . | 25528603 | . | . |
| 3.8L RWD | 1985 | 25524140 | 25518445 | See 1978 |
| . | . | 25520329 | . | . |
| Turbo Regal | . | 25520329 | 25518445 | See 1978 |
| Turbo Riviera | . | 25520231 | 25518445 | See 1978 |
| FWD (Flat lifters) | . | 25520043 | 25518445 | 25514267 |
| 3.8L RWD | 1986 | 25526109 | 25518445 | See 1978 |
| Turbo Regal | . | 25526109 | 25518445 | See 1978 |
| FWD Flat Lifters | . | 25526110 | 25524248 | 25514267 |
| FWD Roller Lifters | . | 25526111 | 25524248 | 25514267 |
| 3.8L RWD | 1987 | 25526109 | 25518445 | See 1978 |
| Turbo Regal | . | 25526109 | 25518445 | See 1978 |
| FWD Roller Lifters | . | 25526110 | 25524248 | 25514267 |
| . | . | 25526111 | 25529889 | 25527515 |
| FWD Roller Lifters | 1988 | 25528801 | 25527515 | 25514267 |
| FWD 3800 Roller | . | 25532617 | 25529723 | 25530502 |
| . | . | . | 25522646 | . |
| 3800 FWD Roller | 1989 | 25528801 | 25527515 | 25514267 |
| Turbo Trans Am | . | 25526109 | 25536702 | 25511229 |
| 3800 FWD Roller | 1990 | 25531617 | 25534250 | 25530502 |
| . | . | . | 25532646 | . |
| 3800 EV6 | . | 25532674 | 25534250 | 25534737 |
| 4.1L RWD | 1968-1972 | 3877178 | 3895052 | . |
| . | . | 3850817 | 3927763 | . |
| 4.1L RWD | 1973-1976 | 366855 | 3962084 | . |
| . | . | 348675 | 14002012 | . |
| . | . | 328575 | 370696 | . |
| . | . | 743483 | 355795 | . |
| . | . | 358675 | 377133 | . |
| 4.1L RWD | 1980-1984 | 25506484 | 25504443 | . |
| . | . | 25505553 | 25505900 | . |
| . | . | 25514291 | 25506293 | . |
| . | . | . | 25518445 |  |