MAF WOT

DIP Switches

| 0 | No Change |
| --- | --- |
| 1 | 2% Rich |
| 2 | 4% Rich |
| 3 | 6% Rich |
| 4 | 8% Rich |
| 5 | 10% Rich |
| 6 | 12% Rich |
| 7 | 14% Rich |
| 8 | not used |
| 9 | 14% Lean |
| A | 12% Lean |
| B | 10% Lean |
| C | 8% Lean |
| D | 6% Lean |
| E | 4% Lean |
| F | 2% Lean |

| - Switch 1 | On |
| --- | --- |
| - Switch 2 | On |
| - Switch 3 | On |
| - Switch 4 | Off |

MAF BASE New Style MAF - Old Style ECM

| 0 | 3.5” LT1/LS1 MAF sensor |
| --- | --- |
| 1 | 3.5” LT1/LS1 MAF sensor (richer at idle) |
| 2 | 3.5” LT1/LS1 MAF sensor (leaner at idle) |

| 0 | No change |
| --- | --- |
| 1 | 2 Degrees Advanced above 3200RPM |
| 2 | 4 Degrees Advanced above 3200RPM |
| 3 | 6 Degrees Advanced above 3200RPM |
| 4 | 8 Degrees Advanced above 3200RPM |
| 5 | 10 Degrees Advanced above 3200RPM |
| 6 | 0 Degrees Retarded to 3200RPM , 3.3 degree Retarded at 4400, 6.6 degrees at 5400, 10 degrees at 6400 |
| 7 | 0 Degrees Retarded to 3200RPM , 2.5 degree Retarded at 4400, 5 degrees at 5400, 7.5 degrees at 6400 |
| 8 | 0 Degrees Retarded to 3200RPM , 2 degree Retarded at 4400, 4 degrees at 5400, 6 degrees at 6400 |
| 9 | 0 Degrees Retarded to 3200RPM , 1.5 degrees Retarded at 4400, 3 degrees at 5400, 4.5 degrees at 6400 |
| A | 0 Degrees Retarded to 3200RPM , 1 degree Retarded at 4400, 2 degrees at 5400, 3 degrees at 6400 |
| B | 10 Degrees Retarded above 3200RPM |
| C | 8 Degrees Retarded above 3200RPM |
| D | 6 Degrees Retarded above 3200RPM |
| E | 4 Degrees Retarded above 3200RPM |
| F | 2 Degrees Retarded above 3200RPM |

| SPARK BASE SPARK WOT 0 | No change |
| --- | --- |
| 1 | 2 Degrees Advanced |
| 2 | 4 Degrees Advanced |
| 3 | 6 Degrees Advanced |
| 4 | 8 Degrees Advanced |
| 5 | 10 Degrees Advanced |
| 6 | 12 Degrees Advanced |
| 7 | 14 Degrees Advanced |
| 8 | not used |
| 9 | 14 Degrees Retarded |
| A | 12 Degrees Retarded |
| B | 10 Degrees Retarded |
| C | 8 Degrees Retarded |
| D | 6 Degrees Retarded |
| E | 4 Degrees Retarded |
| F | 2 Degrees Retarded |