MAF Translator and Translator Plus Version 5 Addendum.

Buick V6 turbo:

Idle calibration has been improved along with filtering for idle stability.  Calibrations are optimized for MAF’s with the screen installed.  The MAF should be installed with the internal divider oriented vertical,  with the electrical connector at the rear.  Many times the MAF’s will be affected by turbulent air from the cooling fan, especially when the air filter is installed directly on the MAF, this effect is worse with the MAF screen removed and will cause shifts in the idle Block Learn values when the fan turns on.  This can be eliminated by using an extension pipe between the MAF and the air filter.

WOT adjustment range has been increased.

LT1 / LS1:

MAF Base range extended:

0 – no change

1 - +5%

2 - +10%

3 - +15%

4 - +20%

5 - +25%       new

6 - -50%       new

7 - -45%       new

8 - -40%       new

9 - -35%

A - -30%

B - -25%

C - -20%

D - -15%

E - -10%

F - -5%

85mm MAF mode:  Setting DipSwitch 4 on enables 85mm MAF mode.  The translator will convert signals from an 85mm MAF (LS6 / Truck / Cadillac) to LS1 calibrated.  By using a retrofit harness a 85mm MAF can be directly substituted for an LS1 MAF.